THE SIX CRITERION AIR POLLUTANTS

| POLLUTANT | # SOURCE | # HEALTH EFFECTS | ENVIRONMENTAL EFFECTS |
| --- | --- | --- | --- |
| Ground-level ozone O3 gaseous | Chemical reaction between nitrogen oxides in vehicular exhaust, VOCs and sunlight | Lung damage Breathing problems Asthma Stuffy nose Eye irritation Reduced resistance to colds and other infections | Major component of smog Damage to plants and trees Reduced visibility Deteriorates rubber and fabric |
| Carbon monoxide CO colorless, odorless gas | Burning of gasoline, wood, natural gas, kerosene, coal, oil, etc. | Headaches, dizziness, confusion, nausea, drowsiness Damage to heart and circulatory system Reduces ability of blood to bring oxygen to cells & tissues in body Death |  |
| ## Sulfur dioxide SO2 gaseous | Burning of diesel and other fuels by vehicles Coal burning by power plants and heating stoves Refineries Industrial processes (making paper & metal) | Respiratory & lung damage Asthma Eye irritation Burning of skin Early death | Reacts to form acid rain Damages plants & trees Damage or kill aquatic life in streams and lakes Eats away concrete and stone structures |
| ## Nitrogen oxides NOx gaseous | Burning of gasoline, natural gas, coal, oil, etc. Vehicles are major source. Power plants burning fossil fuels. | Lung damage Illnesses of respiratory system, including breathing passages and lungs. | Reacts to form acid rain Damages plants & trees Damage or kill aquatic life in streams and lakes Forms ozone & smog Reduces visibility Damages buildings & statues |
| ## Lead Pb metallic element | Vehicles burning leaded gasoline Paint Smelters (metal refineries) Power plants burning fossil fuels. | Brain damage Nervous system damage, especially in children, including hyperactivity Kidney damage Digestive problems Cancer in animals | Contaminates soil Contaminates crops Harms livestock and wildlife |
| Particulate matter Less than 10 microns in size (PM10) Dust, sand, ash, soot, smoke, pollen, exhaust | Burning of wood, coal & diesel fuels Industrial plants Agriculture (plowing, burning of fields) Unpaved roads Construction Mining & quarrying Windblown dust Woodstoves Volcanoes | Lung damage Nose and throat irritation Sneezing Bronchitis Asthma Eye irritation Early death | Reduced visibility Produces haze Dust on leaves can damage crops by preventing plants from carrying out photosynthesis Discolor and dirty buildings, vehicles, statues, clothing, and furniture |